Job Safety Analysis
	JOB SAFETY ANALYSIS RECORD SHEET

	PLANT/EQUIPMENT:

                                Booster Station # 5
	DEPARTMENT:  SACR/CSD
	DATE PREPARED:
Dec. 10,2017
	DATE ISSED:
Dec. 11,2017

	
	M/S Abdul Aziz Ahmed Al Shehri General Contractors ( AASCON)
	JSA NO.:  01

	DESCRIPTION OF JOB/TASK: dismantling/Removal of Low Level Water Pipes

	PREPARED BY:
	REVIEWED BY:
	APPROVED BY:

	REFRENCES:

CSSMS                                                                                                                             Work Permit System GI-2.100

Construction Safety Manual                                                                                      Demolition Job Procedure 

Isolation, LOTO and Use of Hold Tags GI-6.012                                                      SA Safety Handbook

	SAFETY EQUIPMENT REQUIRED:        Safety Hat, Safety Shoes , Safety Gloves, Safety Glasses, Ear Plugs ; pad lock & hold tag ; caution/ warning tape
Note : Adequate and special PPE must also be considered for this task


	JOB STEP NO.
	SEQUENCE OF BASIC JOB STEPS
	POTENTIAL HAZARDS
	REQUIRED HAZARD CONTROLS/SAFE WORK PROCEDURES

	1
	De energization and isolation of Electrical and water supply
	· Electrocution during isolation of electrical supply at the main electrical cabinet.
· Hand and body injury due to stuck up valves.
	· Competent and certified electrician ( ACUD maintenance personnel ) should perform switching off the breaker in the cabinet.
· Lock out tag out by the operation and the contravtors
· Double check with the operation foreman the isolation or pipe blinding



	2
	Dismantling of existing pipes and fittings
	· Fire hazard due to use of oxy/acetylene cutting torch
· Explosion of compresses gas

· Caught in between large pipes during the cutting activity

· Struck by metallic debris during cutting activity

· Caught in between scrap pipes and catwalk during temporary relocation at the site .


	· Provision of Fire extinguishers and fire blanket on site
· Removal and clearing the area from flammable debris before dismantling the pipes

· Provision of fire watch at the job site

· Provision of flashback arrestor at the cutting torch and gauges

· Provision of proper support to the pipe to be cut, using the heavy equipment forklift with the certified operator
· Provision of FRC clothing and proper PPE to the workers

· Properly arrange the dismantled pipes and metallic catwalk to the temporary designated laydown area and provide warning and safety barricades

· All scrap metals shall be lay to the ground and avoid high stock file.



	3
	Loading and transporting the dismantled pipes , calves and fittings .
	· Caught by fallen metal during lifting and loading to the trailer truck
· Fallen scrap materials from the truck during the transportation to contractors yard.


	· Lifting equipment to be operated by certified operator only
· Watch man and rigger shall be at site , inspection heavy equipment and slings prior to use.

· Properly inspect the supported pipes by competent person prior to move or travel the trailer .




